Notes on specific functions used in the TMSAnalysis_Toolbox app

Functions from Fcns\Spike\CEDS64ML folder.

Notes are from the s64mat.pdf file.

In order of appearance

[iOk]=CEDS64LoadLib(sPath)

Loads the ceds64int library allowing the rest of the CEDS64AbcXyz() functions to be used. This function must be called first. It must also be preceded by an addpath command, otherwise MATLAB® will not be able to find the CEDS64LoadLib.m file holding this command.

| sPath | Path to CEDS64ML folder |
| --- | --- |
| iOk | 0 if the operation completed without error, otherwise negative error code |

sPath	Path to CEDS64ML folder

iOk	0 if the operation completed without error, otherwise negative error code

When you are done use unloadlibrary ceds64int to remove the library from memory.

[fhand]=CEDS64Open(sPath{, iMode})

Opens an existing file (there is a limit on the number of files that can be open at once, currently 8).

| SPath | A string holding the path and file name of the file to open. This follows the usual operating system rules for file names. If the file name has the extension .smr, we will first try to open it as a 32-bit SON file. Otherwise we will open it as a 64-bit SON file. |
| --- | --- |
| iMode | If omitted or set to -1, we will attempt to open the file in Read/Write mode, and if this fails, we will open it in ReadOnly mode. Otherwise set 1 to open in ReadOnly mode and 0 for Read/Write mode. |
| fhand | Returned as an integer file handle to the opened file, 0 if no more handles or a negative error code. |

[iChans]=CEDS64MaxChan(fhand)

Return the number of channels that the current file supports. This number was set when the file was created.

| fhand | An integer file handle to identify the file. |
| --- | --- |
| iChans | Returns the maximum number of channels this file can hold or a negative error code . |

[iType]=DEDS64ChanType(fhand, iChan)

This returns an integer code indicating the type of a channel in a file.

| fhand | An integer file handle to identify the file. |
| --- | --- |
| iChan | The channel number (starting with 1). |
| iType | Channel type or a negative error code. |

Channel types

0 no channel

1 Waveform channel

2 Event (falling)

3 Event (rising)

4 Event (both)

5 Marker

6 Wavemark

7 Realmark

8 TextMark

9 Realwave

[iRead, vi64T]=CEDS64readEvents(fhand, iChan, iN, i64From{,i64UpTo{,maskh}})

Read event times from any suitable channel.

You can read event times from any event channel, marker channel or extended marker channel. You can use this to read EventBoth channels, which allows you to select rising or falling events by setting a filter.

| fhand | An integer file handle to identify the file. |
| --- | --- |
| iChan | The channel number (starting at 1). |
| iN | The maximum number of events to return. |
| i64From | The time in ticks to start searching from. This time is included in the range. |
| i64UpTo | Optional, taken as -1 if omitted. The end time of the search, in ticks. If this is set to -1, the search extends to the end of the file. This time is not included in the range. |
| maskh | Optional, no filter if omitted. If present and non-zero, this is a handle to a marker mask with Marker and extended Marker channel types. |
| iRead | Returned as the number of event times read or as a negative error code for use . |
| vi64T | Returned as a vector of 64-bit integers holding event times in ticks. This can be of zero length if no data is returned. |

[i64Div]=CEDS64ChanDiv(fhand,iChan)

This gets the channel divide (sample interval in ticks) for waveform and WaveMarks channels. If used with a channel that does not use a channel divide, the result will be 0.

| fhand | An integer file handle to identify the file. |
| --- | --- |
| iChan | The channel number to be deleted (starting at 1). |
| i64Div | The sample interval in ticks, otherwise a negative error code . |

Ex: Get the channel 1 rate in Hz (assuming that channel 1 is a waveform channel):

SampleRateInHz = 1.0/(CEDS64ChanDiv(fhand, 1)*CEDS64TimeBase(fhand));

[dSecs]=CEDS64TimeBase(fhand{, dNew})

Gets and/or sets the file time base (the length of each tick) for the data file.

| fhand | An integer file handle to identify the file. |
| --- | --- |
| dNew | An optional argument. if present, it sets a new time base value, in seconds per tick. |
| dSecs | If present, returned holding the time base in seconds before any change made by providing dNew. If there is an error, this is returned as a negative error code . |

[i64Time]=CEDS64ChanMaxTime(fhand, iChan)

This function returns the time in ticks of the last item in a channel.

| fhand | An integer file handle to identify the file. |
| --- | --- |
| ichan | The channel number (starting at 1). |
| i64Time | The time of the last item in a channel, or -1 if no items exist, otherwise a negative error code. |

[ iRead, vfWave, i64Time]=CEDS64ReadWaveF( fhand, iChan, iN, i64From{, i64UpTo {, maskh}} )

Read waveform data from waveform, realwave, wavemarker channels as 32-bit floating point values and returns them as a vector. If you read an Adc or WaveMark channel, the values are converted to floats using the scale and offset set for the channel.

The read operation stops at a gap in the data. A gap is defined as an interval between data points that is not the CEDS64ChanDiv() value defined for the channel.

| fhand | An integer file handle to identify the file. |
| --- | --- |
| ichan | The channel number (starting at 1). This must be a suitable channel. |
| iN | The maximum number of events to return. |
| i64From | The time in ticks to start searching from. This time is included in the range |
| i64UpTo | Optional, taken as -1 if omitted. The end time of the read, in ticks. If this is set to -1, the read extends to the end of the file. This time is not included in the range. |
| maskh | Optional, no filter if omitted. If present and non-zero, this is a handle to a marker mask with WaveMark channels. |
| iRead | Returns the number of waveform points read or a negative error code. |
| vfWave | Returns a vector of 32-bit floating point, being the waveform data. This can be of zero length if no data is returned. |
| i64Time | If any data is read, this returns the time in ticks of the first item read into vfWave. If nothing is read, the value should be ignored. |

[ iOk {,sTitle}]=CEDS64ChanTitle( fhand, iChan{,sNew})

Gets and/or sets the channel title. The 64-bit SON library does not impose limits on the length of the title string.

However, the older 32-bit library only allows 9 characters and Spike2 does not expect longer titles so we suggest that you keep them short.

| fhand | An integer file handle to identify the file. |
| --- | --- |
| ichan | The channel number to be deleted (starting at 1). |
| sNew | An optional argument. If present, it set a new channel title. |
| iOk | 0 if the oertation completed correctly, otherwise a negative error code |
| sTitle | If present, returned holding the channel title before any change made by providing sNew |